1.Write a program to print a message using individual letters and the ASCII code of letters.

| Assembly code | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| ; Program: Print "HELLO" using ASCII codes, one letter at a time .MODEL SMALL .STACK 100H .DATA letters DB 72,69,76,76,79 len EQU $-letters .CODE MAIN PROC MOV AX, @DATA MOV DS, AX MOV CX, len LEA SI, letters PRINT_LOOP: MOV DL, [SI] MOV AH, 02H INT 21H INC SI LOOP PRINT_LOOP MOV AH, 4CH INT 21H MAIN ENDP END MAIN | B8 00 00 ; MOV AX, @DATA 8E D8 ; MOV DS, AX B9 05 00 ; MOV CX, 0005h ( BE 00 00 ; LEA SI, letters 8A 14 ; MOV DL, [SI] B4 02 ; MOV AH, 02h CD 21 ; INT 21h 46 ; INC SI E2 F7 ; LOOP PRINT_LOOP B4 4C ; MOV AH, 4Ch CD 21 ; INT 21h 48 45 4C 4C 4F | Reset->DA->B8->+->00+->00->+->cc->STP->GO DA->8E->+->D8->+->cc->STP > GO DA->B9->+->05+->00->+->cc-> STP->GO DA->BE->+->00+->00->+->cc-> STP->GO DA->8A->+->14+->+->cc->STP->GO DA->B4->+->02+->cc->STP->GO DA->cd->+->21+->cc->STP->GO DA->46->+->cc->STP->GO DA->E2->+->F7+->cc->STP->GO DA->B4->+->4C+->cc->STP->GO DA->CD->+->21->+->cc->STP-> GO DA → 4 → +→ 8 → + →STP → GO DA → 4 → +→ 5 → +→STP → GO DA → 4 → +→ C → +→ STP → GO DA → 4 → +→C → +→STP → GO DA → 4 → +→F → +→STP → GO |

2. Write a program to add two numbers as well as form a Fibonacci series.

| Assembly code | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| org 100h .DATA ANS DW ? .CODE MAIN PROC MOV AX,@DATA MOV DS,AX MOV BX,0 MOV DX,1 MOV CX,7 LABEL: ADD BX,DX MOV AX,BX MOV BX,DX MOV DX,AX LOOP LABEL MOV ANS,DX END MAIN ret | B8 ?? ?? ; MOV AX,@DATA depends on data segment address 8E D8 ; MOV DS,AX BB 00 00 ; MOV BX,0 BA 01 00 ; MOV DX,1 B9 07 00 ; MOV CX,7 03 D3 ; ADD BX,DX 89 D8 ; MOV AX,BX 89 DA ; MOV BX,DX 89 D0 ; MOV DX,AX E2 F6 ; LOOP LABEL 89 16 ?? ?? ; MOV [ANS],DX store result C3 ; RET | DA -> B8 ->+-> ?? ->+-> ?? ->+> cc -> STP -> GO DA -> 8E -> +→D8 ->+→ cc -> STP -> GO DA -> BB ->+→ 00 ->+→ 00 ->+→ cc -> STP -> GO DA -> BA ->+→ 01 ->+→ 00 -> +→cc -> STP -> GO DA -> B9 -> +→07 ->+→ 00 -> +→ cc -> STP -> GO DA -> 03 ->+→ D3 -> +→cc -> STP -> GO DA -> 89 -> +→D8 -> +→cc -> STP -> GO DA -> 89 ->+→ DA -> +→cc -> STP -> GO DA -> 89 ->+→ D0 ->+→ cc -> STP -> GO DA -> E2 ->+→ F6 -> +→cc -> STP -> GO DA -> 89 -> +→16 -> +→?? ->+→ ?? ->+→ cc -> STP -> GO DA -> C3 +→-> cc -> STP -> GO |

3. Write a program to exchange the value of AX and BX.

| Assembly Code | Equivalent Machine Code | Flowchart |
| --- | --- | --- |
| ;Write a program to exchange the value of AX and BX. .MODEL SMALL .STACK 100H .DATA num1 DW 1234H ; First number num2 DW 5678H ; Second number .CODE MAIN PROC MOV AX, @DATA MOV DS, AX MOV AX, num1 MOV BX, num2 XCHG AX, BX MOV num1, AX MOV num2, BX MAIN ENDP END MAIN | B8 34 12 ; MOV AX,1234h BB 78 56 ; MOV BX,5678h 87 D8 ; XCHG AX,BX A3 00 00 ; MOV [num1],AX (offset depends) 89 1E 00 00 ; MOV [num2],BX (offset depends) C3 ; RET | Reset -> DA -> B8 -> +34 -> +12 -> cc -> STP -> GO DA -> BB -> +78 -> +56 -> cc -> STP -> GO DA -> 87 -> D8 -> cc -> STP -> GO DA -> A3 -> +00 -> +00 -> cc -> STP -> GO DA -> 89 -> 1E -> +00 -> +00 -> cc -> STP -> GO DA -> C3 -> cc -> STP -> GO |

4. Write a program to find the Square Root of a number.

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA num DW 144 ; input number res DW 0 ; result (square root) .CODE MAIN PROC MOV AX, @DATA MOV DS, AX MOV CX, 0 ; count (result) MOV BX, 1 ; odd number (1,3,5,...) MOV DX, num ; load number into DX L1: SUB DX, BX ; subtract odd number JL ENDLOOP ; if < 0, exit INC CX ; increase result ADD BX, 2 ; next odd number JMP L1 ENDLOOP: MOV res, CX ; store result in res ; Exit program MOV AH, 4CH INT 21H MAIN ENDP END MAIN | B8 00 00 ; MOV AX,@DATA 8E D8 ; MOV DS,AX B9 00 00 ; MOV CX,0 BB 01 00 ; MOV BX,1 BA 90 00 ; MOV DX,num (144 decimal = 90h) 2B D3 ; SUB DX,BX 7C 0C ; JL ENDLOOP 41 ; INC CX 83 C3 02 ; ADD BX,2 EB F3 ; JMP L1 89 0E 00 00 ; MOV res,CX (offset depends on VAR) B4 4C ; MOV AH,4Ch CD 21 ; INT 21h | Reset -> DA -> B8 -> +00 -> +00 -> cc -> STP -> GO DA -> 8E -> D8 -> cc -> STP -> GO DA -> B9 -> +00 -> +00 -> cc -> STP -> GO DA -> BB -> +01 -> +00 -> cc -> STP -> GO DA -> BA -> +90 -> +00 -> cc -> STP -> GO DA -> 2B -> D3 -> cc -> STP -> GO DA -> 7C -> 0C -> cc -> STP -> GO DA -> 41 -> cc -> STP -> GO DA -> 83 -> C3 -> 02 -> cc -> STP -> GO DA -> EB -> F3 -> cc -> STP -> GO DA -> 89 -> 0E -> +00 -> +00 -> cc -> STP -> GO DA -> B4 -> 4C -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO |

5. Write a program  to print a String.

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA msg DB 'Hello, Puspita!$' .CODE MAIN PROC MOV AX, @DATA MOV DS, AX ;---- Print string ---- MOV AH, 9 ; DOS function 9h = print string LEA DX, msg ; DX = string ?? address INT 21H ; interrupt call to print ;---- Exit program ---- MOV AH, 4CH ; DOS terminate program INT 21H MAIN ENDP END MAIN | B8 00 00 ; MOV AX,@DATA (offset depends) 8E D8 ; MOV DS,AX B4 09 ; MOV AH,9 BA 00 00 ; LEA DX,msg (offset depends) CD 21 ; INT 21h B4 4C ; MOV AH,4Ch CD 21 | Reset -> DA -> B8 -> +00 -> +00 -> cc -> STP -> GO ; DA -> 8E -> D8 -> cc -> STP -> GO DA -> B4 -> 09 -> cc -> STP -> GO DA -> BA -> +00 -> +00 -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO DA -> B4 -> 4C -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO |

6. Write a program to subtract two 8-bit BCD numbers.

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA NUM1 DB 45H ; First BCD number (45) NUM2 DB 28H ; Second BCD number (28) RESULT DB ? ; Store result here .CODE MAIN PROC MOV AX, @DATA MOV DS, AX ; Load numbers MOV AL, NUM1 ; AL = first number MOV BL, NUM2 ; BL = second number ; Perform subtraction SUB AL, BL ; AL = AL - BL (raw binary subtraction) DAS ; Decimal Adjust after Subtraction (for BCD correction) ; Store result MOV RESULT, AL ; Exit MOV AH, 4CH INT 21H MAIN ENDP END MAIN | B0 45 ; MOV AL,NUM1 B3 28 ; MOV BL,NUM2 2C D8 ; SUB AL,BL (or 28h) -- depends on assembler 2F ; DAS A2 00 00 ; MOV [RESULT],AL (offset depends) B4 4C ; MOV AH,4Ch CD 21 ; INT 21h | Reset -> DA -> B0 -> +45 -> cc -> STP -> GO DA -> B3 -> +28 -> cc -> STP -> GO DA -> 2C -> D8 -> cc -> STP -> GO ; DA -> 2F -> cc -> STP -> GO DA -> A2 -> +00 -> +00 -> cc -> STP -> GO DA -> B4 -> 4C -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO |

7. Write a program to multiply two 16-bit numbers.

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA NUM1 DW 1234H ; First 16-bit number NUM2 DW 5678H ; Second 16-bit number RESULT1 DW ? ; Low word of result (AX) RESULT2 DW ? ; High word of result (DX) .CODE MAIN PROC MOV AX, @DATA MOV DS, AX ; Load first number into AX MOV AX, NUM1 ; Multiply AX * NUM2 (16-bit) MUL NUM2 ; DX:AX = AX * NUM2 ; Store result in memory (for VAR tab observation) MOV RESULT1, AX ; Lower 16 bits MOV RESULT2, DX ; Upper 16 bits ; Exit program MOV AH, 4CH INT 21H MAIN ENDP END MAIN | A1 34 12 ; MOV AX,[NUM1] (offset depends) F7 26 78 56 ; MUL WORD [NUM2] -> DX:AX A3 00 00 ; MOV [RESULT1], AX A3 02 00 ; MOV [RESULT2], DX B4 4C ; MOV AH,4Ch CD 21 | Reset -> DA -> A1 -> +34 -> +12 -> cc -> STP -> GO DA -> F7 -> 26 -> +78 -> +56 -> cc -> STP -> GO DA -> A3 -> +00 -> +00 -> cc -> STP -> GO DA -> A3 -> +02 -> +00 -> cc -> STP -> GO DA -> B4 -> 4C -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO |

8. Write a program to subtract two 16-bit numbers with or without borrow.

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA NUM1 DW 1234H ; First 16-bit number NUM2 DW 0ABCDH ; Second 16-bit number RESULT DW ? ; Store 16-bit result here .CODE MAIN PROC MOV AX, @DATA MOV DS, AX MOV AX, NUM1 ; Load first number into AX SUB AX, NUM2 ; Subtract NUM2 from AX MOV RESULT, AX ; Store result in RESULT MOV AH, 4CH ; Exit program INT 21H MAIN ENDP END MAIN | A1 34 12 ; MOV AX,[NUM1] (depends on offset) 2B 16 CD 0A ; SUB AX,[NUM2] (depends on offset) A3 00 00 ; MOV [RESULT],AX B4 4C ; MOV AH,4Ch CD 21 ; INT 21h | Reset -> DA -> A1 -> +34 -> +12 -> cc -> STP -> GO DA -> 2B -> 16 -> +CD -> +0A -> cc -> STP -> GO DA -> A3 -> +00 -> +00 -> cc -> STP -> GO DA -> B4 -> 4C -> cc -> STP -> GO DA -> CD -> 21 -> cc -> STP -> GO |

9. Write a program to add two 8-bit BCD numbers

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| org 100h .DATA num1 DB 75h ; first BCD number (75 decimal) num2 DB 37h ; second BCD number (37 decimal) result DB ? ; store BCD result carryFlag DB ? ; store carry flag if overflow .CODE start: MOV AL, num1 ; load first BCD number ADD AL, num2 ; add second BCD number DAA ; decimal adjust AL MOV result, AL ; store result ; store carry flag if needed JC setCarry MOV carryFlag, 0 JMP done setCarry: MOV carryFlag, 1 done: RET END start | B0 75 ; MOV AL, num1 04 37 ; ADD AL, num2 27 ; DAA A2 00 00 ; MOV [result], AL (depends on offset) 72 02 ; JC setCarry C6 06 00 00 00 ; MOV byte [carryFlag],0 EB 02 ; JMP done C6 06 00 00 01 ; MOV byte [carryFlag],1 C3 | Reset -> DA -> B0 -> +75 -> cc -> STP -> GO DA -> 04 -> +37 -> cc -> STP -> GO DA -> 27 -> cc -> STP -> GO DA -> A2 -> +00 -> +00 -> cc -> STP -> GO DA -> 72 -> 02 -> cc -> STP -> GO DA -> C6 -> 06 -> +00 -> +00 -> 00 -> cc -> STP -> GO ; DA -> EB -> 02 -> cc -> STP -> GO DA -> C6 -> 06 -> +00 -> +00 -> 01 -> cc -> STP -> GO DA -> C3 -> cc -> STP -> GO |

10. Write a program  for Binary to Decimal Conversion

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| org 100h .DATA binaryNum db hundreds db tens db ? ; tens digit units db ? ; units digit .CODE start: mov al, binaryNum xor ah, ah ; clear AH for 16-bit division mov bl, 100 ; divisor = 100 div bl ; AX / BL -> AL=quotient, AH=remainder mov hundreds, al ; store quotient = hundreds mov al, ah ; move remainder back to AL xor ah, ah ; clear AH mov bl, 10 ; divisor = 10 div bl ; AL / 10 -> AL=quotient, AH=remainder mov tens, al ; store tens digit mov units, ah ; remainder = units digit ret END start | ; A0 00 00 ; MOV AL, [binaryNum] ; 30 E4 ; XOR AH,AH ; B3 64 ; MOV BL,64h (100 decimal) ; F6 F3 ; DIV BL ; A2 00 00 ; MOV [hundreds],AL ; 8A C4 ; MOV AL,AH ; 30 E4 ; XOR AH,AH ; B3 0A ; MOV BL,0Ah (10 decimal) ; F6 F3 ; DIV BL ; A2 00 00 ; MOV [tens],AL ; 88 26 00 00 ; MOV [units] , AH ; C3 ; RET | Reset -> DA -> A0 -> +00 -> +00 -> cc -> STP -> GO DA -> 30 -> E4 -> cc -> STP -> GO DA -> B3 -> +64 -> cc -> STP -> GO DA -> F6 -> F3 -> cc -> STP -> GO DA -> A2 -> +00 -> +00 -> cc -> STP -> GO DA -> 8A -> C4 -> cc -> STP -> GO DA -> 30 -> E4 -> cc -> STP -> GO DA -> B3 -> +0A -> cc -> STP -> GO DA -> F6 -> F3 -> cc -> STP -> GO DA -> A2 -> +00 -> +00 -> cc -> STP -> GO DA -> 88 -> 26 -> +00 -> +00 -> cc -> STP -> GO DA -> C3 -> cc -> STP -> GO |

11. Write a program  to find the factorial of a number.

| Assembly code | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| .MODEL SMALL .STACK 100H .DATA NUM DB 05H ; Number (n = 5 ? factorial = 120) RESULT DW 01H ; Result initialized to 1 (16-bit, since factorial grows fast) .CODE MAIN PROC MOV AX, @DATA MOV DS, AX ; Load number into CL (counter) MOV CL, NUM MOV AX, 1 ; AX = RESULT = 1 FACTORIAL_LOOP: MUL CL ; AX = AX * CL (unsigned multiply) DEC CL ; CL = CL - 1 JNZ FACTORIAL_LOOP ; Repeat until CL = 0 MOV RESULT, AX ; Store final factorial in RESULT ; Exit program MOV AH, 4CH INT 21H MAIN ENDP END MAIN | B8 ?? ?? ; MOV AX, @DATA 8E D8 ; MOV DS, AX B1 05 ; MOV CL, NUM B8 01 00 ; MOV AX, 1 F6 E1 ; FACTORIAL_LOOP: MUL CL FE C9 ; FE C9 FE C9 ; JNZ FACTORIAL_LOOP A3 ?? ?? ; MOV RESULT, AX B4 4C ; MOV AH, 4CH CD 21 ; INT 21H |  |

12. Write a program  for Decimal to Binary Conversion

| Assembly Language | Equivalent Machine code | Flowchart |
| --- | --- | --- |
| org 100h .DATA decimalNum db 173 ; decimal number to convert .CODE start: mov al, decimalNum ; load the decimal number mov cl, 8 ; number of bits to display mov bl, 80h ; mask starting at 10000000b (MSB) print_bit: mov ah, 0Eh ; BIOS teletype function mov dl, al and dl, bl ; isolate current bit jz zero mov dl, '1' jmp display zero: mov dl, '0' display: int 10h ; print character shr bl, 1 ; move mask to next bit (MSB ? LSB) dec cl jnz print_bit ret | A0 00 00 ; MOV AL,[decimalNum] B1 08 ; MOV CL,08h B3 80 ; MOV BL,80h B4 0E ; MOV AH,0Eh 8A D0 ; MOV DL,AL 22 D3 ; AND DL,BL 74 02 ; JZ zero B2 31 ; MOV DL,'1' EB 02 ; JMP display B2 30 ; MOV DL,'0' CD 10 ; INT 10h D0 EB ; SHR BL,1 FE C9 ; DEC CL 75 F0 ; JNZ print_bit C3 ; RET | Reset -> DA -> A0 -> +00 -> +00 -> cc -> STP -> GO ; DA -> B1 -> +08 -> cc -> STP -> GO ; DA -> B3 -> +80 -> cc -> STP -> GO DA -> B4 -> +0E -> cc -> STP -> GO DA -> 8A -> D0 -> cc -> STP -> GO DA -> 22 -> D3 -> cc -> STP -> GO DA -> 74 -> 02 -> cc -> STP -> GO DA -> B2 -> +31 -> cc -> STP -> GO DA -> EB -> 02 -> cc -> STP -> GO DA -> B2 -> +30 -> cc -> STP -> GO DA -> CD -> 10 -> cc -> STP -> GO DA -> D0 -> EB -> cc -> STP -> GO DA -> FE -> C9 -> cc -> STP -> GO DA -> 75 -> F0 -> cc -> STP -> GO DA -> C3 -> cc -> STP -> GO |

13. Write a program to add two 16-bit numbers.

| Assembly Code | Machine Equivalent Code | Flowchart |
| --- | --- | --- |
| ;16bit addition with carry org 100h .DATA num1 DW 1234h ; first 16-bit number num2 DW 0ABCDh ; second 16-bit number result DW ? ; store sum carryFlag DB ? ; store carry flag if overflow .CODE start: MOV AX, num1 ; load first number ADD AX, num2 ; add second number MOV result, AX ; store result ; check carry flag manually JC carry_set MOV carryFlag, 0 JMP done carry_set: MOV carryFlag, 1 done: RET END start | A1 00 00 ; MOV AX, [num1] (depends on offset) 03 16 00 00 ; ADD AX, [num2] (depends on offset) A3 00 00 ; MOV [result], AX (depends on offset) 72 04 ; JC carry_set (short jump) C6 06 00 00 00 ; MOV byte [carryFlag], 0 (depends on offset) EB 02 ; JMP done (short jump) C6 06 00 00 01 ; MOV byte [carryFlag], 1 (depends on offset) C3 ; RET | Reset -> DA -> A1 -> +34 -> +12 -> cc -> STP -> GO ; MOV AX,[num1] ; DA -> 03 -> 16 -> +CD -> +0A -> cc -> STP -> GO ; ADD AX,[num2] ; DA -> A3 -> +00 -> +00 -> cc -> STP -> GO ; MOV [result],AX ; DA -> 72 -> 04 -> cc -> STP -> GO ; JC carry_set ; DA -> C6 -> 06 -> +00 -> +00 -> 00 -> cc -> STP -> GO ; MOV [carryFlag],0 ; DA -> EB -> 02 -> cc -> STP -> GO ; JMP done ; DA -> C6 -> 06 -> +00 -> +00 -> 01 -> cc -> STP -> GO ; MOV [carryFlag],1 ; DA -> C3 -> cc -> STP -> GO |

14.Equation solve

| Assembly Code | Machine Equivalent Code | Flowchart |
| --- | --- | --- |
| org 100h .DATA res_f DW ? ; store 3X^2 + 2X -5 res_g DW ? ; store 2X^2 -1 X_val DW 0Eh ; X = 14 decimal .CODE MAIN PROC ; Compute X^2 MOV AX, X_val MOV BX, X_val MUL BX ; AX = X^2 MOV [2008], AX ; temp store X^2 ; Compute 3X^2 MOV BX, 3 MOV AX, [2008] MUL BX ; AX = 3X^2 MOV [2002], AX ; Compute 3X^2 + 2X -5 MOV AX, X_val MOV BX, 2 MUL BX ; AX = 2X ADD AX, [2002] ; AX = 3X^2 + 2X SUB AX, 5 ; AX = 3X^2 + 2X -5 MOV [res_f], AX ; Compute 2X^2 -1 MOV AX, [2008] ; AX = X^2 MOV BX, 2 MUL BX ; AX = 2X^2 SUB AX, 1 MOV [res_g], AX ; Divide f/g MOV AX, [res_f] ; numerator MOV BX, [res_g] ; denominator XOR DX, DX ; clear DX before DIV DIV BX ; AX = quotient, DX = remainder RET MAIN ENDP END MAIN | B8 0E 00 ; MOV AX, X_val BB 0E 00 ; MOV BX, X_val F7 E3 ; MUL BX A3 08 20 ; MOV [2008], AX BB 03 00 ; MOV BX, 3 A1 02 20 ; MOV AX, [2002] F7 E3 ; MUL BX A3 02 20 ; MOV [2002], AX B8 0E 00 ; MOV AX, X_val BB 02 00 ; MOV BX, 2 F7 E3 ; MUL BX 03 C2 ; ADD AX, [2002] 2D 05 00 ; SUB AX, 5 A3 0A 20 ; MOV [res_f], AX A1 08 20 ; MOV AX, [2008] BB 02 00 ; MOV BX, 2 F7 E3 ; MUL BX 2D 01 00 ; SUB AX, 1 A3 0C 20 ; MOV [res_g], AX A1 0A 20 ; MOV AX, [res_f] A3 0C 20 ; MOV BX, [res_g] 31 D2 ; XOR DX, DX F7 F3 ; DIV BX | Reset -> DA -> B8 -> +0E -> +00 -> cc -> STP -> GO DA -> BB -> +0E -> +00 -> cc -> STP -> GO DA -> F7 -> E3 -> cc -> STP -> GO DA -> A3 -> +08 -> +20 -> cc -> STP -> GO DA -> BB -> +03 -> +00 -> cc -> STP -> GO DA -> A1 -> +02 -> +20 -> cc -> STP -> GO DA -> F7 -> E3 -> cc -> STP -> GO MUL BX DA -> A3 -> +02 -> +20 -> cc -> STP -> GO DA -> B8 -> +0E -> +00 -> cc -> STP -> GO DA -> BB -> +02 -> +00 -> cc -> STP -> GO DA -> F7 -> E3 -> cc -> STP -> GO DA -> 03 -> C2 -> cc -> STP -> GO DA -> 2D -> 05 -> +00 -> cc -> STP -> GO DA -> A3 -> +0A -> +20 -> cc -> STP -> GO DA -> A1 -> +08 -> +20 -> cc -> STP -> GO DA -> BB -> +02 -> +00 -> cc -> STP -> GO DA -> F7 -> E3 -> cc -> STP -> GO DA -> 2D -> 01 -> +00 -> cc -> STP -> GO DA -> A3 -> +0C -> +20 -> cc -> STP -> GO DA -> A1 -> +0A -> +20 -> cc -> STP -> GO DA -> A3 -> +0C -> +20 -> cc -> STP -> GO DA -> 31 -> D2 -> cc -> STP -> GO DA -> F7 -> F3 -> cc -> STP -> GO |